# Animation Steps – Quick Guide

Setup (all files): Upload the STEP file to Fusion 360 and open it. Switch the workspace to ANIMATION. All directions below use the model’s own axes: Z is up. Distances are approximate, so anything that clearly separates the parts is fine.

Core tools:

- Transform Components moves or rotates parts. Use Set Pivot and click a round edge to rotate about its centre.

- Auto Explode spreads everything out automatically.

- Restore Home puts everything back.

- Each action starts at the playhead on the timeline. Drag the action bars to change timing.

## 00 Starter – Jar with Lid (3 parts)

- Lid – rotate 2 turns (720°) about Z and move up 10 mm in the same action to unscrew it.

- Lid: move up 70 mm.

- Lid Seal: move up 40 mm.

- Restore Home to screw it back on.

Learn: move vs rotate, Set Pivot, the timeline.

## 01 Simple – Spur Gear Reduction (17 parts)

- Gears in mesh: rotate Pinion + Input Shaft +360° and Driven Gear + Output Shaft −180° over the same 4 s. The teeth should stay meshed, showing a 2:1 ratio and opposite rotation.

- New storyboard, teardown:

  - Shaft collars ±Y 100.

  - Shafts slide out ±Y 130.

  - Both gears up 150.

  - Bearings ±Y 45.

  - Cap screws down 45.

  - Brackets up 80.

Learn: simultaneous actions, rotation ratios, a logical disassembly order.

## 02 Moderate – Game Controller (22 parts, 2 sub-assemblies)

- Screws (×5): move down 40 mm from the underside.

- Front Shell Assembly: move up 90 mm. The buttons and D-pad go with it because they belong to this sub-assembly.

- Inside the front shell assembly, push the D-pad and buttons A/B/X/Y up 30 mm.

- Main Board Assembly (PCB with thumbsticks): move up 45 mm. Then lift the Thumbstick Caps up a further 30 mm.

- Battery up 15 mm, Rumble Motors up 25 mm.

- Triggers: rotate about 30° about their pivot pin (X axis), then slide them back 40 mm (+Y).

Try: compare Auto Explode: One Level with All Levels to see how sub-assemblies behave. Learn: sub-assemblies, selecting inside a sub-assembly in the Browser.

## 03 Advanced – Inline-4 Engine (141 parts, nested)

Storyboard 1, teardown (hide removed parts with the eye icon to keep the view clear):

- Drain plug down, then the ignition coils and oil filler cap up.

- Valve cover up, then the spark plugs up.

- Crank pulley bolt and crank pulley off, then the timing belt forward (−X).

- Intake manifold −Y, exhaust manifold +Y, oil filter −Y (rotate it as well to unscrew).

- Head bolts up, then the Cylinder Head Assembly up, then the head gasket up.

- Oil pan down.

- Rod caps and bolts down, then each Piston and Rod Assembly up out of the bores.

- Main cap bolts and main caps down, then the Crankshaft Assembly down.

Storyboard 2, timing: hide the block, head and covers. Rotate the Crankshaft Assembly 720° about X while the Camshaft + Cam Pulley rotate 360° over the same 8 s. This shows the 2:1 crank-to-cam ratio.

Discuss: the pistons don’t follow the crank, because Animation ignores joints. Motion Study in the Design workspace can show linked motion.

## 04 Extra – Piston and Connecting Rod (9 parts)

- Rod Bolts down 120 mm.

- Rod Cap down 70 mm.

- Gudgeon Pin slides out sideways 90 mm (Y).

- Piston up 90 mm.

- Oil ring, then the compression rings, up off the piston one at a time (110–150 mm).

Learn: a quick exploded view, plus a callout naming each part.

Finish every file by adding a Callout or two, then Publish Video.